TV Mounting Bracket

INLETS

BY: ASHAR AHMED JAVED

PURPOSE:

The purpose of this study was to evaluate the structural integrity of the engineered design, as well as to determine the elements of potential failure.

This study utilizes a provided design and FEA-Finite Element Analysis software was used to identify undesirable feature characteristics through computer simulation. This document presents chronologically the process of identification of engineered features.

Material: Acrylic (MPa Threshold 139.043 ≤)

| Model name: TV Bracket Current Configuration: FEA |
| --- |
| Solid Bodies |  |  |  |
| Document Name and Reference | Treated As | Volumetric Properties | Document Path/Date Modified |
| Fillet2 | Solid Body | Mass:0.185214 kg Volume:0.000132295 m^3 Density:1,400 kg/m^3 Weight:1.81509 N | D:\New Life\Learning SOLIDWORKS\Vertanux1 Simulations\Lab 1\TV Bracket.SLDPRT Mar 17 17:25:58 2025 |

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 3.8147e-05; 311.375; 9.53674e-06; 311.375. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; 70 lbf..

ANALYSIS – A: F.O.S

TV MOUNTING BRACKET (ORIGINAL DESIGN)

Results: The TV Mounting Bracket seems to be over engineered.

During analysis we discovered the 70 LBF torque with a material yield strength of MPa 60 exceeds the given requirements by a factor of 10 times the desired outcome. This would lead us to believe the part as-is is over engineered.

SUMMARY: PASS – NOTE: OVER ENGINEERED BY A FACTOR OF 10x, OPTIMIZATION SHOULD BE EXAMINIED TO REDUCE MATERIAL COST.

| Name | Type | Min | Max |
| --- | --- | --- | --- |
| Factor of Safety | Automatic | 2.000e+00 Node: 1 | 2.000e+00 Node: 1 |
| TV Bracket-Static-Factor of Safety-Factor of Safety1 |  |  |  |

ANALYSIS – B: Von Mises

TV MOUNTING BRACKET STRESS ANALYSIS (REVISED/OPTIMIZED DESIGN)

Results: The TV Mounting Bracket appears to be designed well above expectation with regards to yield by being almost four times stronger than required. This may be over engineered.

SUMMARY: PASS

| Name | Type | Min | Max |
| --- | --- | --- | --- |
| Stress1 | VON: von Mises Stress | 5.840e-03N/mm^2 (MPa) Node: 2563 | 1.508e+01N/mm^2 (MPa) Node: 179 |
| TV Bracket-Static-Stress-Stress1 |  |  |  |

ANALYSIS – C

TV MOUNTING BRACKET DISPLACEMENT ANALYSIS

Results: The TV Mounting Bracket bends in the downward direction approximately by 4 mm, which may indicate a stiffer material should be considered.

SUMMARY: PASS

| Name | Type | Min | Max |
| --- | --- | --- | --- |
| Displacement1 | URES: Resultant Displacement | 0.000e+00mm Node: 1 | 3.960e-01mm Node: 597 |
| TV Bracket-Static-Displacement-Displacement1 |  |  |  |